ad5592r_reg_write(dev, AD5592R_REG_CTRL, 0x20)

AD5592R_REG_CTRL = 0x3h

b0 to b3 reserved

b4 = 0 => DAC output range is 0 to V_ref

b5 = 1 => ADC gain is 0 to 2xV_ref (NOT DEFAULT!!)

b6 = 0 => for future DAC writes, the DAC addres bits determine which DAC is written to

b7 = 0 => the contents of the I/O_x pin configuration register can be changed

b8 = 0 =>  ADC buffer is disabled

b9 = 0 => ADC buffer is not used to precharge the adc

ad5592r_reg_write(dev, AD5592R_REG_DAC_EN, 0x0);

AD5592R_REG_DAC_EN = 0x5h

B9=1 => selects the I/O pin 7 as output

B7 to B0 = 00h => the I/O pin function is determined by the pin configuration registers

ad5592r_reg_write(dev, AD5592R_REG_ADC_EN, 0xff);

Asta e configuration register

AD5592R_REG_ADC_EN		= 0x4

B7…B0 = ffh => selects I/O pins 7 to 0 as ADC input

ad5592r_reg_write(dev, AD5592R_REG_ADC_SEQ, 0xff);

AD5592R_REG_ADC_SEQ		= 0x2,

B7…B0 = ffh => includes ADC channels 7 to 0 in conversion sequence

B8 = 0 => Disable temperature indicator readback.

B9 = 0 => Sequence repetition disabled

ad5592r_reg_write(dev, AD5592R_REG_PD, 0x200);

AD5592R_REG_PD			= 0xB

B7…B0 = 00h => channels 7 to 0 operate in normal mode

B9 = 1 => the reference and it’s buffer are powered up

B10 = 0 => the reference and DACs power down states are determined by EN_REF and PD7 to PD0 bits.

ad5592r_reg_read(dev, AD5592R_REG_PD,&temp_reg_val);

AD5592R_REG_PD			= 0xB

The second transaction is a noop